MAESTRO

Modular Ontology Stack for Future-Proof Manufacturing

Technical Overview & Module Reference

Version 0.3.0

Author: Melwin Xavier

Date: 2026-06-15

Audience: stakeholder handover

# Contents

Right-click and choose “Update Field” to build the table of contents.

# 1. Executive summary

MAESTRO is a modular, standards-aligned ontology for Industry 4.0/5.0 manufacturing. It describes a manufacturing system as a knowledge graph (RDF/OWL) so that software — including reasoners and AI agents — can answer questions such as “can this plant make this product?” and “which resources and skills are needed?” without any vendor-specific glue code.

Its central idea is an inversion of the traditional Machine → Function model. Instead of hard-coding what a machine does, MAESTRO separates four concerns: a Capability (what is possible, vendor-neutral), a Skill (the logical behaviour that realises it), an Execution adapter (the technology that implements the skill — ROS, PLC, IEC 61499, OPC UA), and a Resource (the physical asset). Because these are decoupled, you can swap a robot, change a controller, or expose a skill as a service without rewriting the model.

This document describes MAESTRO version 0.3.0: its nine-layer architecture, each of the 24 ontology modules with its current classes and fields, the reasoning pipeline, a worked example, and the planned future work for validation and extension.

# 2. Architecture overview — the nine-layer stack

MAESTRO is organised as a nine-layer semantic stack. Each layer talks only to the layer immediately below it, and each layer is replaceable in isolation. From top to bottom: Product (what is made), Process (how), Capability (what the plant can do), Skill (logical behaviour), Orchestration (the SPARQL/SHACL/LLM reasoning fabric), Execution (ROS, PLC, IEC 61499, OPC UA, AAS), Resource (physical machines), Motion (kinematic primitives), and the Physical Factory itself.

Figure 1 — The nine-layer semantic stack.

Three boundaries do most of the work and are the reason the stack stays future-proof:

Capability ↔ Skill — separates what is possible (vendor-neutral intent) from how it is achieved (logical behaviour).

Skill ↔ Execution — separates technology-neutral behaviour from technology-specific implementation (ROS / PLC / IEC 61499 / OPC UA).

Resource ↔ Motion — separates the physical asset from the geometric and kinematic semantics of how it moves.

# 3. The modular stack and import graph

The vocabulary is split into 24 small TTL modules grouped by concern and tied together by a single umbrella file, maestro.ttl, which imports them all. Every module imports only what it needs (most depend just on core), which keeps the stack easy to extend: a new concern is a new file plus one owl:imports line. All modules share the permanent namespace base https://w3id.org/maestro/ and carry version 0.3.0.

Figure 2 — The 24 modules and the umbrella import graph.

# 4. Module-by-module reference

Each module below lists its purpose, its namespace prefix and IRI, and the classes, properties and notable individuals it currently defines. Tables are generated directly from the TTL source, so they reflect the live ontology.

## 4.1  Core

The single root every other module builds on.

### Manufacturing Core  (core:)

File: ontologies/core/manufacturing-core.ttl    Namespace: https://w3id.org/maestro/core#

The root abstractions of the whole stack. Everything is an Entity, split into PhysicalEntity and LogicalEntity. The five manufacturing pillars — Resource, Skill, Capability, Process, Product — and the universal relations that bind them (provides, realizes, requiresCapability, implements, controls) live here.

Figure — Manufacturing Core (10-class-alignment.png).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| core:Capability | core:LogicalEntity | A manufacturing possibility (what a plant CAN do). Realised by Skills. |
| core:Constraint | core:LogicalEntity | A restriction on Skills, Resources, or Processes. |
| core:ControlComponent | core:LogicalEntity | A software/hardware element that implements Skills and controls Resources (e.g. ROS node, PLC FB, IEC 61499 FB, OPC UA method). |
| core:Entity | — | The most general class. Anything modelled in MAESTRO is an Entity. |
| core:Event | core:LogicalEntity | A point-in-time occurrence in the manufacturing system. |
| core:LogicalEntity | core:Entity | An entity that exists only in the information model. |
| core:PhysicalEntity | core:Entity | An entity that has a physical realisation (mass, location). |
| core:Plant | core:PhysicalEntity | An aggregation of Resources forming a manufacturing facility. |
| core:Process | core:LogicalEntity | Manufacturing logic. Requires Capabilities to be executed. |
| core:Product | core:LogicalEntity | A product, part, or assembly that requires manufacturing Processes. |
| core:Resource | core:PhysicalEntity | Any physical manufacturing asset (machine, robot, sensor, actuator, tool). |
| core:Skill | core:LogicalEntity | A logical, executable behaviour. Implemented by ControlComponents; provided by Resources. |
| core:State | core:LogicalEntity | A discrete operational condition of a Resource or ControlComponent. |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| core:canManufacture | Object | core:Plant → core:Product | Inferred relation: a Plant can manufacture a Product. Materialised by rules/manufacturing-ability.rq. |
| core:canPerform | Object | core:Resource → core:Capability | Inferred relation: a Resource is capable of performing a Capability. Materialised by rules/capability-inference.rq. |
| core:communicatesThrough | Object | core:ControlComponent → — | A ControlComponent communicates with a Resource through a communication channel. Range is com:Channel (kept open here to avoid pulling communication.ttl into core). |
| core:composedOfSkill | Object | core:Skill → core:Skill | A composite Skill is composed of another Skill. Transitive. |
| core:connectedTo | Object | core:Entity → core:Entity | Physical or logical connectivity between two entities. Symmetric. |
| core:consumes | Object | core:Entity → core:Event | An Entity consumes an Event. |
| core:controls | Object | core:ControlComponent → core:Resource | A ControlComponent controls a Resource. |
| core:dependsOn | Object | core:Entity → core:Entity | Generic dependency relation. |
| core:deployedOn | Object | core:ControlComponent → core:Resource | A logical ControlComponent is deployed on a physical controller or compute Resource. |
| core:description | Data | core:Entity → xsd:string |  |
| core:emits | Object | core:Entity → core:Event | An Entity emits an Event. |
| core:executes | Object | core:ControlComponent → — | A ControlComponent executes a Process or Skill at runtime. Range is core:Skill or core:Process — left open via union (not asserted to keep OWL2-RL clean). |
| core:hasPart | Object | core:Entity → core:Entity | Mereological part-of relation. Transitive: Plant hasPart Cell, Cell hasPart Robot entails Plant hasPart Robot. |
| core:hasPlantCapability | Object | core:Plant → core:Capability | Plant-level capability. Use this instead of core:canPerform on Plant individuals (canPerform is Resource-scoped). |
| core:hasState | Object | core:Entity → core:State | Runtime operational State of an Entity. Use only inside a runtime named graph — see decisions/0002-runtime-graph-separation.md. |
| core:hosts | Object | core:Resource → core:LogicalEntity | A physical Resource hosts a logical artefact (a ControlComponent, an IEC 61499 RuntimeResource, an OPC UA Server). |
| core:identifier | Data | core:Entity → xsd:string | Globally unique identifier (e.g. AAS shortId, OPC UA NodeId). Functional (each entity has at most one) and inverse-functional (each identifier names at most one entity). |
| core:implementedBy | Object | core:Skill → core:ControlComponent | Inverse of core:implements. |
| core:implements | Object | core:ControlComponent → core:Skill | A ControlComponent implements a Skill (concrete realisation). |
| core:partOf | Object | core:Entity → core:Entity | Inverse mereological relation. Transitive. |
| core:providedBy | Object | core:Skill → core:Resource | Inverse of core:provides. |
| core:provides | Object | core:Resource → core:Skill | A Resource provides a Skill (offers it for use). |
| core:realizedBySkill | Object | core:Capability → core:Skill | A Capability is realized by a reusable Skill specification. |
| core:realizes | Object | core:Skill → core:Capability | A Skill realises a Capability. |
| core:requiresCapability | Object | core:Process → core:Capability | A Process requires a Capability. |
| core:requiresMotion | Object | core:Skill → core:LogicalEntity | A Skill requires a technology-neutral motion specification. Range narrowed to motion:Motion in motion.ttl. |
| core:version | Data | core:Entity → xsd:string |  |

## 4.2  Physical layer

What physically exists on the shop floor and how it moves.

### Resource  (res:)

File: ontologies/physical/resource.ttl    Namespace: https://w3id.org/maestro/resource#

Physical manufacturing assets: machines, robots and cobots, conveyors, CNC and laser units, tools and grippers, actuators, and the controller hardware (PLC, industrial PC) that hosts control components.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| res:Actuator | core:Resource |  |
| res:AGV | res:Machine | Automated Guided Vehicle. |
| res:AssemblyStation | res:Machine | Workstation where assembly operations take place. |
| res:CNC | res:Machine | Computer Numerical Control machining centre. |
| res:Cobot | res:Robot | A collaborative robot designed for safe human interaction. |
| res:ControllerDevice | core:Resource | Physical controller or compute hardware that hosts logical ControlComponents. |
| res:Conveyor | res:Machine | Material handling conveyor. |
| res:Gripper | res:Tool |  |
| res:IndustrialPC | res:ControllerDevice | Industrial compute hardware that hosts logical ControlComponents. |
| res:LaserUnit | res:Machine | Laser machining / marking / welding unit. |
| res:Machine | core:Resource | A general manufacturing machine. |
| res:PLC | res:ControllerDevice | Programmable Logic Controller hardware. PLC programs and Function Blocks are logical ControlComponents deployed on this hardware. |
| res:Robot | res:Machine | An articulated, programmable, multi-axis machine. |
| res:ServoMotor | res:Actuator |  |
| res:Tool | core:Resource |  |
| res:ToolChanger | res:Tool |  |
| res:TransferArm | res:Robot | A robotic arm specialised for transfer / pick-and-place tasks. |
| res:VacuumUnit | res:Tool | Vacuum end-effector for pick operations. |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| res:manufacturer | Data | — → xsd:string |  |
| res:model | Data | — → xsd:string |  |
| res:payload | Object | res:Robot → unit:Quantity | Maximum payload as an explicit quantity value. |
| res:payloadKg | Data | — → xsd:float | Maximum payload of a robot or handling resource, in kilograms. |
| res:reach | Object | res:Robot → unit:Quantity | Robot reach as an explicit quantity value. |
| res:reachMm | Data | — → xsd:float | Robot reach in millimetres. |
| res:serialNumber | Data | — → xsd:string |  |

### Motion  (motion:)

File: ontologies/physical/motion.ttl    Namespace: https://w3id.org/maestro/motion#

Technology-neutral geometric and kinematic primitives — linear, rotational, joint, Cartesian, synchronized, trajectory and force-controlled motion — kept separate from the resources that execute them.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| motion:Axis | core:LogicalEntity |  |
| motion:CartesianMotion | motion:Motion | Motion specified in Cartesian/task space. |
| motion:CoordinateFrame | core:LogicalEntity |  |
| motion:ForceControlledMotion | motion:Motion | Motion regulated by force/torque feedback. |
| motion:JointMotion | motion:Motion | Motion specified in joint space. |
| motion:LinearMotion | motion:Motion |  |
| motion:Motion | core:LogicalEntity | A geometric/kinematic movement abstraction. |
| motion:RotationalMotion | motion:Motion |  |
| motion:SynchronizedMotion | motion:Motion | Coordinated motion of multiple resources. |
| motion:TrajectoryMotion | motion:Motion | Motion defined by a parametric trajectory. |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| motion:hasAcceleration | Data | — → xsd:float |  |
| motion:hasCoordinateFrame | Object | — → motion:CoordinateFrame |  |
| motion:hasPrecision | Data | — → xsd:float | Repeatability / positioning precision in mm. |
| motion:hasVelocity | Data | — → xsd:float | Linear velocity in m/s or angular velocity in rad/s, depending on motion subtype. |
| motion:numberOfAxes | Data | — → xsd:integer |  |
| motion:requiresAxis | Object | — → motion:Axis |  |

## 4.3  Logical layer

The vendor-neutral semantics: what is made, how, and what it takes.

### Skill  (skill:)

File: ontologies/logical/skill.ttl    Namespace: https://w3id.org/maestro/skill#

The keystone of the stack. A Skill is reusable logical behaviour — it is NOT a ROS node, a PLC function block or an OPC UA method; execution technologies implement skills. Atomic skills are primitives; composite skills are built from other skills via composedOfSkill.

Figure — Skill (03-skill-hierarchy.png).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| skill:AssemblySkill | skill:CompositeSkill |  |
| skill:AtomicSkill | core:Skill | An indivisible skill primitive. |
| skill:CompositeSkill | core:Skill | A skill built by composing other skills via skill:composedOfSkill. |
| skill:CoordinationSkill | skill:CompositeSkill |  |
| skill:HandlingSkill | skill:AtomicSkill | VDI 2860 handling skills (orient, position, deposit, secure). |
| skill:InspectionSkill | skill:AtomicSkill |  |
| skill:ManipulationSkill | skill:AtomicSkill | Skills that change the spatial relation between a resource and a workpiece (grasp, pick, place, release). |
| skill:MotionSkill | skill:AtomicSkill | Skills that produce kinematic motion (move linear, move joint, …). |
| skill:ProcessSkill | skill:CompositeSkill |  |
| skill:TransferSkill | skill:CompositeSkill | A composite skill: pick at A, move to B, place at B. |
| skill:WeldingSkill | skill:CompositeSkill |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| skill:composedOfSkill | Object | core:Skill → core:Skill | A composite Skill has another Skill as a constituent step. |
| skill:expectedDurationSeconds | Data | core:Skill → xsd:float |  |
| skill:hasParameter | Object | core:Skill → — |  |
| skill:hasPostcondition | Object | core:Skill → core:State |  |
| skill:hasPrecondition | Object | core:Skill → core:State |  |
| skill:requiresMotion | Object | core:Skill → motion:Motion | A Skill requires a reusable technology-neutral Motion specification. |

### Capability  (cap:)

File: ontologies/logical/capability.ttl    Namespace: https://w3id.org/maestro/capability#

What a plant can do, expressed vendor-neutrally (TransportCapability, not MoveLeftCapability). Capabilities are organised as a SKOS concept scheme and are realised by one or more Skills.

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| cap:realizedBySkill | Object | core:Capability → core:Skill | A Capability specification is realized by a reusable Skill specification. |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| cap:AssemblyCapability | core:Capability, skos:Concept | Ability to combine parts into an assembly. |
| cap:CapabilityScheme | skos:ConceptScheme | SKOS organization for reusable manufacturing capability specifications. |
| cap:HandlingCapability | core:Capability, skos:Concept | Handling capability |
| cap:InspectionCapability | core:Capability, skos:Concept | Inspection capability |
| cap:MachiningCapability | core:Capability, skos:Concept | Machining capability |
| cap:PackagingCapability | core:Capability, skos:Concept | Packaging capability |
| cap:PickPlaceCapability | core:Capability, skos:Concept | Pick a part at location A and place it at location B. |
| cap:TestingCapability | core:Capability, skos:Concept | Testing capability |
| cap:TransportCapability | core:Capability, skos:Concept | Ability to transport material between locations. |
| cap:WeldingCapability | core:Capability, skos:Concept | Welding capability |

### Process  (proc:)

File: ontologies/logical/process.ttl    Namespace: https://w3id.org/maestro/process#

The manufacturing process taxonomy aligned to DIN 8580 (primary shaping, forming, separating, joining, coating, property change) plus handling/transport/inspection, with sequencing relations (precedes, follows, parallelWith).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| proc:AssemblyProcess | proc:JoiningProcess |  |
| proc:Coating | proc:ManufacturingProcess | DIN 8580 Group 5. |
| proc:Forming | proc:ManufacturingProcess | DIN 8580 Group 2 — bending, drawing, forging. |
| proc:HandlingProcess | proc:ManufacturingProcess |  |
| proc:InspectionProcess | proc:ManufacturingProcess |  |
| proc:JoiningProcess | proc:ManufacturingProcess | DIN 8580 Group 4 — welding, screwing, gluing. |
| proc:MachiningProcess | proc:Separating |  |
| proc:ManufacturingProcess | core:Process |  |
| proc:MaterialPropertyChange | proc:ManufacturingProcess | DIN 8580 Group 6 — heat treatment. |
| proc:PrimaryShaping | proc:ManufacturingProcess | DIN 8580 Group 1 — casting, sintering. |
| proc:Separating | proc:ManufacturingProcess | DIN 8580 Group 3 — cutting, machining. |
| proc:TransportProcess | proc:HandlingProcess |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| proc:follows | Object | proc:ManufacturingProcess → proc:ManufacturingProcess |  |
| proc:parallelWith | Object | proc:ManufacturingProcess → proc:ManufacturingProcess |  |
| proc:precedes | Object | proc:ManufacturingProcess → proc:ManufacturingProcess |  |
| proc:producesState | Object | proc:ManufacturingProcess → core:State |  |
| proc:requiresCapability | Object | proc:ManufacturingProcess → core:Capability |  |

### Product  (prod:)

File: ontologies/logical/product.ttl    Namespace: https://w3id.org/maestro/product#

Products, parts and assemblies, plus the features, tolerances and materials that drive which processes a product requires.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| prod:Assembly | core:Product | A Product composed of multiple Parts and/or sub-Assemblies. |
| prod:Feature | core:LogicalEntity | A geometric or functional feature of a Product (hole, slot, surface). |
| prod:Material | core:LogicalEntity |  |
| prod:Part | core:Product | An atomic manufactured item — not further decomposed in this model. |
| prod:Tolerance | core:LogicalEntity | A dimensional, geometric, or surface tolerance specification. |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| prod:hasFeature | Object | core:Product → prod:Feature |  |
| prod:hasTolerance | Object | prod:Feature → prod:Tolerance |  |
| prod:madeOf | Object | core:Product → prod:Material |  |
| prod:mass | Object | core:Product → unit:Quantity | Product mass as a unit-bearing quantity. |
| prod:massKg | Data | core:Product → xsd:float | Deprecated unit-suffixed datatype property. Use prod:mass with unit:Quantity instead. |
| prod:requiresProcess | Object | core:Product → proc:ManufacturingProcess | A Product requires a Manufacturing Process to be produced. |

## 4.4  Execution adapters

Technology-specific bindings that implement logical skills.

### ROS / ROS 2  (ros:)

File: ontologies/execution/ros.ttl    Namespace: https://w3id.org/maestro/ros#

ROS nodes, topics, services, actions, parameters, TF frames and MoveIt controllers. ROS nodes implement skills and control resources.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| ros:MoveItController | ros:ROSNode |  |
| ros:ROSAction | core:LogicalEntity |  |
| ros:ROSNode | core:ControlComponent |  |
| ros:ROSParameter | core:LogicalEntity |  |
| ros:ROSService | core:LogicalEntity |  |
| ros:ROSTopic | core:LogicalEntity |  |
| ros:TFFrame | core:LogicalEntity | A coordinate frame published on the ROS /tf tree. |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| ros:advertisesAction | Object | ros:ROSNode → ros:ROSAction |  |
| ros:advertisesService | Object | ros:ROSNode → ros:ROSService |  |
| ros:controlsResource | Object | ros:ROSNode → core:Resource |  |
| ros:implementsSkill | Object | ros:ROSNode → core:Skill |  |
| ros:publishes | Object | ros:ROSNode → ros:ROSTopic |  |
| ros:rosVersion | Data | — → xsd:string | ROS distribution / version (e.g. 'humble', 'iron', 'jazzy'). |
| ros:subscribes | Object | ros:ROSNode → ros:ROSTopic |  |

### IEC 61131-3 (PLC)  (plc:)

File: ontologies/execution/iec61131.ttl    Namespace: https://w3id.org/maestro/iec61131#

Classic PLC world: programs, tasks, function blocks, functions and variables, with the five IEC 61131-3 languages (LD, ST, FBD, SFC, IL) as individuals.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| plc:Function | core:ControlComponent |  |
| plc:FunctionBlock | core:ControlComponent | An IEC 61131-3 Function Block (FB). |
| plc:Language | — |  |
| plc:PLCProgram | core:ControlComponent |  |
| plc:Task | core:LogicalEntity |  |
| plc:Variable | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| plc:controlsResource | Object | plc:FunctionBlock → core:Resource |  |
| plc:cycleTimeMs | Data | plc:Task → xsd:float |  |
| plc:implementsSkill | Object | plc:FunctionBlock → core:Skill |  |
| plc:writtenIn | Object | — → plc:Language |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| plc:FunctionBlockDiagram | plc:Language |  |
| plc:InstructionList | plc:Language |  |
| plc:LadderLogic | plc:Language |  |
| plc:SequentialFunctionChart | plc:Language |  |
| plc:StructuredText | plc:Language |  |

### IEC 61499  (iec61499:)

File: ontologies/execution/iec61499.ttl    Namespace: https://w3id.org/maestro/iec61499#

Event-driven distributed control: basic, composite and service-interface function blocks, applications, runtime resources and devices. Kept disjoint from IEC 61131 function blocks.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| iec61499:Application | core:LogicalEntity |  |
| iec61499:BasicFB | iec61499:FunctionBlock | A basic function block defined by an ECC (Execution Control Chart). |
| iec61499:CompositeFB | iec61499:FunctionBlock | A composite function block built from interconnected FBs. |
| iec61499:Connection | core:LogicalEntity |  |
| iec61499:Device | core:Resource | A physical computing device that hosts IEC 61499 RuntimeResources. Subsumed under core:Resource (controller hardware) and aligned with res:ControllerDevice. |
| iec61499:ExecutionEvent | core:Event | An IEC 61499 execution event (formerly iec61499:Event in 0.2.0). Renamed to avoid collision with core:Event. |
| iec61499:FunctionBlock | core:ControlComponent | IEC 61499 function block. Disjoint with plc:FunctionBlock (IEC 61131-3 FB) to prevent accidental dual-typing. |
| iec61499:RuntimeResource | core:LogicalEntity | IEC 61499 runtime resource (formerly iec61499:Resource in 0.2.0). Renamed to avoid collision with core:Resource. |
| iec61499:ServiceInterfaceFB | iec61499:FunctionBlock |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| iec61499:consumesEvent | Object | iec61499:FunctionBlock → iec61499:ExecutionEvent |  |
| iec61499:controlsResource | Object | iec61499:FunctionBlock → core:Resource |  |
| iec61499:emitsEvent | Object | iec61499:FunctionBlock → iec61499:ExecutionEvent |  |
| iec61499:hostsRuntimeResource | Object | iec61499:Device → iec61499:RuntimeResource |  |
| iec61499:implementsSkill | Object | iec61499:FunctionBlock → core:Skill |  |

### OPC UA  (opcua:)

File: ontologies/execution/opcua.ttl    Namespace: https://w3id.org/maestro/opcua#

OPC UA servers, nodes, objects, variables and methods, plus a SkillInterface pattern that exposes a MAESTRO Skill over the network.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| opcua:OpcUaMethod | opcua:OpcUaNode |  |
| opcua:OpcUaNode | core:LogicalEntity |  |
| opcua:OpcUaObject | opcua:OpcUaNode |  |
| opcua:OpcUaServer | core:ControlComponent |  |
| opcua:OpcUaSkillInterface | opcua:OpcUaObject | An OPC UA object that exposes a MAESTRO Skill to network clients. Mirrors the CaSkMan SkillInterface pattern. |
| opcua:OpcUaVariable | opcua:OpcUaNode |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| opcua:accessibleThrough | Object | core:Skill → opcua:OpcUaSkillInterface | A Skill is accessible through an OPC UA SkillInterface (inverse of opcua:exposes). |
| opcua:browseName | Data | opcua:OpcUaNode → xsd:string |  |
| opcua:exposes | Object | opcua:OpcUaSkillInterface → core:Skill | An OPC UA SkillInterface exposes a Skill. |
| opcua:hostsInterface | Object | opcua:OpcUaServer → opcua:OpcUaSkillInterface |  |
| opcua:implementsSkill | Object | opcua:OpcUaMethod → core:Skill |  |
| opcua:nodeId | Data | opcua:OpcUaNode → xsd:string | OPC UA NodeId (e.g. 'ns=2;s=Skill.Transfer.Start'). |

### Asset Administration Shell  (aas:)

File: ontologies/execution/aas.ttl    Namespace: https://w3id.org/maestro/aas#

The Industry 4.0 digital-twin envelope: administration shells, assets, submodels and submodel elements (properties, operations).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| aas:Asset | core:PhysicalEntity |  |
| aas:AssetAdministrationShell | core:LogicalEntity | The digital twin envelope of an Asset. |
| aas:Operation | aas:SubmodelElement |  |
| aas:Property | aas:SubmodelElement |  |
| aas:Reference | core:LogicalEntity |  |
| aas:Submodel | core:LogicalEntity |  |
| aas:SubmodelElement | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| aas:hasSubmodel | Object | aas:AssetAdministrationShell → aas:Submodel |  |
| aas:hasSubmodelElement | Object | aas:Submodel → aas:SubmodelElement |  |
| aas:idShort | Data | — → xsd:string |  |
| aas:representsAsset | Object | aas:AssetAdministrationShell → aas:Asset |  |
| aas:semanticId | Data | — → xsd:string | ECLASS / IEC CDD identifier. |

## 4.5  Runtime layer

Live operational state, kept apart from design-time models.

### Runtime  (runtime:)

File: ontologies/runtime/runtime.ttl    Namespace: https://w3id.org/maestro/runtime#

Live behaviour: runtime events, state transitions and snapshots, with PackML-aligned operational states (Available, Busy, Executing, Held, Fault, …) as individuals.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| runtime:RuntimeEvent | core:Event |  |
| runtime:Snapshot | core:LogicalEntity | Marker class for a runtime snapshot ontology / named graph. New in 0.3.0. |
| runtime:StateTransition | runtime:RuntimeEvent |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| runtime:enteredState | Object | runtime:StateTransition → core:State |  |
| runtime:fromState | Object | runtime:StateTransition → core:State |  |
| runtime:snapshotTaken | Data | runtime:Snapshot → xsd:dateTime | Wall-clock timestamp at which the snapshot was captured. |
| runtime:timestamp | Data | runtime:RuntimeEvent → xsd:dateTime |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| runtime:Aborted | core:State, skos:Concept | Aborted |
| runtime:Alarm | core:State, skos:Concept | Alarm |
| runtime:Available | core:State, skos:Concept | Resource is ready to accept work. |
| runtime:Busy | core:State, skos:Concept | Resource is currently executing a skill. |
| runtime:Executing | core:State, skos:Concept | PackML 'Execute' substate. |
| runtime:Fault | core:State, skos:Concept | Resource is in a faulted or error state. |
| runtime:HealthState | core:State, skos:Concept | Diagnostic health indicator such as healthy, degraded, or failed. |
| runtime:Held | core:State, skos:Concept | PackML 'Held' state: execution paused. |
| runtime:Idle | core:State, skos:Concept | Idle |
| runtime:StateScheme | skos:ConceptScheme |  |

### State machine  (state:)

File: ontologies/runtime/state.ttl    Namespace: https://w3id.org/maestro/state#

A generic state-machine vocabulary — state machines, transitions, guards, initial and final states — reused across resources and control components.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| state:FinalState | core:State |  |
| state:InitialState | core:State |  |
| state:StateMachine | core:LogicalEntity |  |
| state:Transition | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| state:guardExpression | Data | state:Transition → xsd:string |  |
| state:hasStateMachine | Object | — → state:StateMachine |  |
| state:hasTransition | Object | state:StateMachine → state:Transition |  |
| state:source | Object | state:Transition → core:State |  |
| state:target | Object | state:Transition → core:State |  |

## 4.6  Cross-cutting concerns

Orthogonal aspects that apply across every layer.

### Units & quantities  (unit:)

File: ontologies/cross-cutting/unit.ttl    Namespace: https://w3id.org/maestro/unit#

QUDT-aligned units and unit-bearing quantities, so physical values carry explicit units instead of bare floats.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| unit:Quantity | core:LogicalEntity, qudt:Quantity |  |
| unit:Unit | core:LogicalEntity, qudt:Unit |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| unit:hasUnit | Object | — → unit:Unit |  |
| unit:numericValue | Data | — → xsd:double |  |
| unit:symbol | Data | unit:Unit → xsd:string |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| unit:Degree | unit:Unit |  |
| unit:DegreeCelsius | unit:Unit |  |
| unit:Joule | unit:Unit |  |
| unit:Kelvin | unit:Unit |  |
| unit:Kilogram | unit:Unit |  |
| unit:Metre | unit:Unit |  |
| unit:MetrePerSecond | unit:Unit |  |
| unit:Millimetre | unit:Unit |  |
| unit:Newton | unit:Unit |  |
| unit:Pascal | unit:Unit |  |
| unit:Radian | unit:Unit |  |
| unit:Second | unit:Unit |  |
| unit:Watt | unit:Unit |  |

### Sensors  (sensor:)

File: ontologies/cross-cutting/sensor.ttl    Namespace: https://w3id.org/maestro/sensor#

W3C SOSA/SSN-aligned sensors and observations, with industrial sensor types (temperature, pressure, vibration, force, torque, proximity, vision).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| sensor:FeatureOfInterest | core:LogicalEntity, sosa:FeatureOfInterest |  |
| sensor:ObservableProperty | core:LogicalEntity, sosa:ObservableProperty |  |
| sensor:Observation | core:Event, sosa:Observation | A sensor observation. Subclass of core:Event so it joins the MAESTRO eventing model. |
| sensor:Sensor | core:Resource, sosa:Sensor | Physical sensor. Subclass of both core:Resource and sosa:Sensor so SOSA-native graphs federate cleanly. |

### Communication  (com:)

File: ontologies/cross-cutting/communication.ttl    Namespace: https://w3id.org/maestro/communication#

Protocols, channels and endpoints — OPC UA, DDS, MQTT, PROFINET, EtherCAT, Modbus, HTTP — that connect control components.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| com:Channel | core:LogicalEntity | A bidirectional communication channel using a Protocol. |
| com:Endpoint | core:LogicalEntity |  |
| com:Protocol | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| com:address | Data | com:Endpoint → xsd:string | URI/URL of the endpoint (e.g. 'opc.tcp://plc1:4840'). |
| com:hasEndpoint | Object | com:Channel → com:Endpoint |  |
| com:port | Data | com:Endpoint → xsd:integer |  |
| com:usesProtocol | Object | com:Channel → com:Protocol |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| com:DdsProtocol | com:Protocol |  |
| com:EthercatProtocol | com:Protocol |  |
| com:HttpProtocol | com:Protocol |  |
| com:ModbusProtocol | com:Protocol |  |
| com:MqttProtocol | com:Protocol |  |
| com:OpcUaProtocol | com:Protocol |  |
| com:ProfinetProtocol | com:Protocol |  |

### Safety  (safety:)

File: ontologies/cross-cutting/safety.ttl    Namespace: https://w3id.org/maestro/safety#

Functional safety: safety functions, hazards and risks, with IEC 61508 SIL and ISO 13849 Performance Level individuals, and emergency stop.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| safety:EmergencyStop | safety:SafetyFunction |  |
| safety:Hazard | core:LogicalEntity |  |
| safety:PerformanceLevel | — | ISO 13849 Performance Level (PL a … PL e). |
| safety:Risk | core:LogicalEntity |  |
| safety:SafetyFunction | core:LogicalEntity | A function intended to maintain or achieve a safe state (IEC 61508). |
| safety:SafetyIntegrityLevel | — | IEC 61508 SIL (SIL1 … SIL4). |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| safety:hasPerformanceLevel | Object | safety:SafetyFunction → safety:PerformanceLevel |  |
| safety:hasSIL | Object | safety:SafetyFunction → safety:SafetyIntegrityLevel |  |
| safety:mitigates | Object | safety:SafetyFunction → safety:Hazard |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| safety:PLa | safety:PerformanceLevel |  |
| safety:PLb | safety:PerformanceLevel |  |
| safety:PLc | safety:PerformanceLevel |  |
| safety:PLd | safety:PerformanceLevel |  |
| safety:PLe | safety:PerformanceLevel |  |
| safety:SIL1 | safety:SafetyIntegrityLevel |  |
| safety:SIL2 | safety:SafetyIntegrityLevel |  |
| safety:SIL3 | safety:SafetyIntegrityLevel |  |
| safety:SIL4 | safety:SafetyIntegrityLevel |  |

### Quality  (qa:)

File: ontologies/cross-cutting/quality.ttl    Namespace: https://w3id.org/maestro/quality#

Quality checks, defects, non-conformances, inspection results (Pass/Fail/Rework) and KPIs (OEE, FPY, scrap rate).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| qa:Defect | core:LogicalEntity |  |
| qa:InspectionResult | core:LogicalEntity |  |
| qa:KPI | core:LogicalEntity | Key performance indicator (OEE, FPY, scrap rate, …). |
| qa:NonConformance | core:Event |  |
| qa:QualityCheck | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| qa:detected | Object | qa:QualityCheck → qa:Defect |  |
| qa:hasResult | Object | qa:QualityCheck → qa:InspectionResult |  |
| qa:kpiValue | Data | qa:KPI → xsd:double |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| qa:Fail | qa:InspectionResult |  |
| qa:Pass | qa:InspectionResult |  |
| qa:Rework | qa:InspectionResult |  |

### Maintenance  (maint:)

File: ontologies/cross-cutting/maintenance.ttl    Namespace: https://w3id.org/maestro/maintenance#

Preventive, corrective, predictive and inspection maintenance actions, plus fault diagnosis and reliability metrics (RUL, MTBF).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| maint:Corrective | maint:MaintenanceAction |  |
| maint:FaultDiagnosis | core:LogicalEntity |  |
| maint:Inspection | maint:MaintenanceAction |  |
| maint:MaintenanceAction | core:Event |  |
| maint:Predictive | maint:MaintenanceAction | Maintenance triggered by predictive analytics over sensor data. |
| maint:Preventive | maint:MaintenanceAction |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| maint:mtbfHours | Data | core:Resource → xsd:double | Mean time between failures, in hours. |
| maint:performedAt | Data | maint:MaintenanceAction → xsd:dateTime |  |
| maint:remainingUsefulLifeHours | Data | core:Resource → xsd:double |  |
| maint:scheduledFor | Data | maint:MaintenanceAction → xsd:dateTime |  |

### Energy  (energy:)

File: ontologies/cross-cutting/energy.ttl    Namespace: https://w3id.org/maestro/energy#

Energy consumers, measurements and sources, with power, energy and carbon-intensity fields for energy-aware manufacturing.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| energy:EnergyConsumer | core:Resource |  |
| energy:EnergyMeasurement | core:Event |  |
| energy:EnergySource | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| energy:carbonGCO2eqPerKwh | Data | energy:EnergySource → xsd:double | Grams CO2-equivalent emitted per kWh produced. |
| energy:energyKwh | Data | energy:EnergyMeasurement → xsd:double |  |
| energy:powerWatts | Data | energy:EnergyConsumer → xsd:double |  |
| energy:suppliedBy | Object | energy:EnergyConsumer → energy:EnergySource |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| energy:CompressedAir | energy:EnergySource |  |
| energy:ElectricalGrid | energy:EnergySource |  |
| energy:Solar | energy:EnergySource |  |
| energy:Wind | energy:EnergySource |  |

## 4.7  Reasoning & planning

How the graph is reasoned over, and where it sits in the enterprise.

### Reasoning  (reas:)

File: ontologies/reasoning/reasoning.ttl    Namespace: https://w3id.org/maestro/reasoning#

Documents the layered reasoning stack (semantic inheritance via OWL2-RL, validation via SHACL, capability inference and orchestration via SPARQL, explanation via LLM) and the canonical reasoning chain.

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| reas:ReasoningLayer | core:LogicalEntity |  |
| reas:ReasoningModel | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| reas:technology | Data | reas:ReasoningLayer → xsd:string |  |

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| reas:CapabilityInference | reas:ReasoningLayer |  |
| reas:CoreReasoningChain | reas:ReasoningModel | Product →’ requires →’ Process →’ requires →’ Capability →’ realizedBy →’ Skill →’ implementedBy →’ ControlComponent →’ controls →’ Resource →’ performs →’ Motion. |
| reas:Explanation | reas:ReasoningLayer |  |
| reas:Orchestration | reas:ReasoningLayer |  |
| reas:SemanticInheritance | reas:ReasoningLayer |  |
| reas:Validation | reas:ReasoningLayer |  |

### Planning (ISA-95)  (plan:)

File: ontologies/reasoning/planning.ttl    Namespace: https://w3id.org/maestro/planning#

ISA-95 enterprise hierarchy (enterprise, site, area, work centre, work unit) and production planning objects (orders, schedules, work orders).

Classes

| Class | Subclass of | Description |
| --- | --- | --- |
| plan:Area | core:LogicalEntity |  |
| plan:Enterprise | core:LogicalEntity |  |
| plan:OperationsDefinition | core:LogicalEntity |  |
| plan:ProductionOrder | core:LogicalEntity |  |
| plan:ProductionSchedule | core:LogicalEntity |  |
| plan:Site | core:LogicalEntity |  |
| plan:WorkCenter | core:LogicalEntity |  |
| plan:WorkOrder | core:LogicalEntity |  |
| plan:WorkUnit | core:LogicalEntity |  |

Properties

| Property | Kind | Domain → Range | Description |
| --- | --- | --- | --- |
| plan:assignedTo | Object | plan:WorkOrder → plan:WorkCenter |  |
| plan:dueDate | Data | plan:ProductionOrder → xsd:dateTime |  |
| plan:partOfArea | Object | plan:WorkCenter → plan:Area |  |
| plan:partOfSite | Object | plan:Area → plan:Site |  |
| plan:quantity | Data | plan:ProductionOrder → xsd:integer |  |

## 4.8  Reusable libraries

Canonical instance data with stable, citable IRIs.

### Skill library  (skill-lib:)

File: ontologies/lib/skill-lib.ttl    Namespace: https://w3id.org/maestro/skill-lib#

Reusable skill individuals (MoveLinear, MoveJoint, VacuumPick, Release, Transfer) separated from the skill: vocabulary so application data can cite stable IRIs.

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| skill-lib:MoveJoint | skill:MotionSkill | Move the controlled point through configured joint space waypoints. |
| skill-lib:MoveLinear | skill:MotionSkill | Move the controlled point along a straight line at constant velocity. |
| skill-lib:Release | skill:ManipulationSkill | Release a held part by deactivating the holding mechanism (e.g. vacuum off, gripper open). |
| skill-lib:Transfer | skill:TransferSkill | Composite pick-and-place skill — moves a part from A to B. |
| skill-lib:VacuumPick | skill:ManipulationSkill | Activate vacuum, lower onto part, lift. |

### Motion library  (motion-lib:)

File: ontologies/lib/motion-lib.ttl    Namespace: https://w3id.org/maestro/motion-lib#

Canonical reusable motion specifications (linear, joint, Cartesian) that skills point at via requiresMotion.

Individuals

| Individual | Type | Description |
| --- | --- | --- |
| motion-lib:CartesianMotionSpec | motion:CartesianMotion | Canonical reusable specification for Cartesian motion requirements. |
| motion-lib:JointMotionSpec | motion:JointMotion | Canonical reusable specification for joint-space motion requirements. |
| motion-lib:LinearMotionSpec | motion:LinearMotion | Canonical reusable specification for linear motion requirements. |

# 5. Reasoning and the LLM pipeline

MAESTRO reasons over the graph in layers: OWL2-RL handles semantic inheritance, SHACL validates the data, SPARQL CONSTRUCT rules infer capabilities, and SPARQL queries drive orchestration. The canonical reasoning chain threads the whole stack together:

Product → Process → Capability → Skill → ControlComponent → Resource → Motion

Figure 3 — The core reasoning chain.

On top of this, a Large Language Model translates a natural-language question into SPARQL, runs it against the graph database, and explains the deterministic result. The LLM never stores facts — it only translates intent and narrates answers; all knowledge lives in the graph.

Figure 4 — The LLM + graph-database reasoning pipeline.

# 6. Standards alignment

MAESTRO does not reinvent existing standards; it federates with them so models stay interoperable and vendor-independent.

Figure 5 — External standards MAESTRO aligns with.

| Standard | Role in MAESTRO | Module(s) |
| --- | --- | --- |
| DIN 8580 | Manufacturing process taxonomy | process |
| VDI 2860 | Handling skill categories | skill |
| VDI 3682 | Process sequencing | process |
| IEC 61131-3 | Classic PLC programming model | iec61131 |
| IEC 61499 | Event-driven distributed control | iec61499 |
| OPC UA | Interoperable skill interfaces | opcua |
| Asset Administration Shell | Digital-twin envelope | aas |
| ISA-95 | Enterprise / planning hierarchy | planning |
| PackML | Operational state model | runtime |
| IEC 61508 / ISO 13849 | Functional safety (SIL / PL) | safety |
| QUDT | Units and quantities | unit |
| W3C SOSA / SSN | Sensors and observations | sensor |
| W3C SKOS | Capability & state concept schemes | capability, runtime |

# 7. Worked example — the Distribution Station

The Festo MPS Distributing Station is modelled end-to-end as the reference example. Its plant topology, skill bridge and reasoning pipeline are all expressed with the same modules described above.

Figure 6 — Distribution Station plant topology.

Figure 7 — Bridging resource-side skill types to library skills.

Figure 8 — Distribution Station reasoning pipeline.

Figure 9 — The three-layer (design / reasoning / runtime) view.

Verified result: ex:DistributingStation1 core:canManufacture ex:WorkpieceWP3

This conclusion is inferred — not asserted — from a live GraphDB session of roughly 5,500 triples using OWL2-RL plus two SPARQL CONSTRUCT rules.

# 8. Future works

## 8.1  Validation roadmap (Festo MPS stations)

The framework is being validated against Festo MPS reference stations in three milestones. Milestone numbers (M1/M2/M3) are independent of the code release version.

| Milestone | Station | Focus | Query class enabled |
| --- | --- | --- | --- |
| M1 (v0.1) | Distributing Station | Base stack: 16 modules, plant + product + runtime; largely complete. | Basic feasibility — canManufacture, canPerform |
| M2 (v0.2) | Distributing Station | Constant-speed motion sub-classes for timing; safety (SIL) instances. | Time-aware — cycle-time, scheduling, SIL filtering |
| M3 (v0.3) | Drilling Station | Second station for portability; AAS digital twin; machine operations. | Gap identification — find missing capabilities |

Each milestone closes with new example files, new SPARQL queries, and a git tag (v0.1-roadmap-m1 … v0.3-roadmap-m3). Open questions for review include whether security belongs in M2 or M3, whether the Drilling Station is a physical or model-only target, and confirming the M1/M2/M3 naming alongside semantic version tags.

## 8.2  Industry 5.0 extensions

MAESTRO is designed so the next generation of manufacturing concepts can be added as new modules rather than patches. The current stack already accommodates:

| Extension | How MAESTRO supports it today |
| --- | --- |
| Autonomous factories | The machine-readable reasoning chain lets a planner traverse it without human guidance. |
| AI manufacturing agents | Agents talk to SPARQL, not directly to PLCs; intent extraction is separated from deterministic reasoning. |
| Self-reconfiguration | Skills are not bound to resources; swapping an end-effector re-fires capability inference. |
| Manufacturing-as-a-Service | OPC UA SkillInterfaces expose skills network-wide for remote invocation. |
| Semantic digital twins | aas + runtime cover the AAS envelope plus live behaviour. |
| Predictive maintenance | maintenance carries RUL, MTBF and a dedicated Predictive action class. |
| Energy-aware manufacturing | energy exposes power, energy and carbon intensity by source. |
| Human-robot collaboration | Cobot + safety (SIL/PL) describe the safety envelope; runtime states make it explicit. |

# 9. Glossary — namespace prefixes

| Prefix | Namespace IRI | Module |
| --- | --- | --- |
| core: | https://w3id.org/maestro/core# | manufacturing-core.ttl |
| res: | https://w3id.org/maestro/resource# | resource.ttl |
| motion: | https://w3id.org/maestro/motion# | motion.ttl |
| skill: | https://w3id.org/maestro/skill# | skill.ttl |
| cap: | https://w3id.org/maestro/capability# | capability.ttl |
| proc: | https://w3id.org/maestro/process# | process.ttl |
| prod: | https://w3id.org/maestro/product# | product.ttl |
| plc: | https://w3id.org/maestro/iec61131# | iec61131.ttl |
| iec61499: | https://w3id.org/maestro/iec61499# | iec61499.ttl |
| ros: | https://w3id.org/maestro/ros# | ros.ttl |
| opcua: | https://w3id.org/maestro/opcua# | opcua.ttl |
| aas: | https://w3id.org/maestro/aas# | aas.ttl |
| runtime: | https://w3id.org/maestro/runtime# | runtime.ttl |
| state: | https://w3id.org/maestro/state# | state.ttl |
| sensor: | https://w3id.org/maestro/sensor# | sensor.ttl |
| unit: | https://w3id.org/maestro/unit# | unit.ttl |
| com: | https://w3id.org/maestro/communication# | communication.ttl |
| safety: | https://w3id.org/maestro/safety# | safety.ttl |
| qa: | https://w3id.org/maestro/quality# | quality.ttl |
| maint: | https://w3id.org/maestro/maintenance# | maintenance.ttl |
| energy: | https://w3id.org/maestro/energy# | energy.ttl |
| reas: | https://w3id.org/maestro/reasoning# | reasoning.ttl |
| plan: | https://w3id.org/maestro/planning# | planning.ttl |
| skill-lib: | https://w3id.org/maestro/skill-lib# | skill-lib.ttl |
| motion-lib: | https://w3id.org/maestro/motion-lib# | motion-lib.ttl |