# MAESTRO — Festo Distributing Station Explainer

A walk-through of the TTL model, the reasoning chain, and the live GraphDB session

Melwin Xavier

2026-05-26

Table of Contents

# MAESTRO — Festo Distributing Station

## A walk-through of the TTL model, the reasoning chain, and the live GraphDB session

Author: Melwin Xavier Repository: maestro-ds on the local GraphDB instance Branch: usecase_festo Date: 2026-05-26

## 1. Why this document exists

The Festo Distributing Station (DS) is the smallest MPS station that still contains every modelling concern MAESTRO is designed to capture:

- a physical plant decomposed into machines, actuators, sensors, and an end-effector,

- logical skills registered at runtime to a 4DIAC / Schneider EAE application (Comp_Skill_Adp),

- product / process demand that needs to be matched against the plant's abilities,

- a runtime snapshot (which resource is Available right now), and

- a reasoning chain that turns all of the above into the single answer the orchestrator cares about: "can this plant manufacture this workpiece right now?".

This file explains every TTL block in the examples/distribution-station/ folder, links each block to the ontology module it depends on, and shows the live GraphDB session in which the three TTL files were loaded, the OWL2-RL ruleset was activated, and the two SPARQL-CONSTRUCT inference rules (capability-inference.rq, manufacturing-ability.rq) were materialised. It closes with a worked example of a single SPARQL SELECT that simultaneously joins explicit triples, OWL/RDFS-inferred triples, and custom-rule-derived triples — the headline property of the architecture.

## 2. The figures referenced in this document

Figure 1 — Festo Distributing Station plant topology. ex:DistributingStation1 (a core:Plant) is decomposed via core:hasPart into the FeederUnit and the TransferUnit, each further decomposed into actuators, end-effectors and sensors. The material-flow link ex:feeds lifts to core:connectedTo through subproperty inference.

Figure 2 — Modular ontology stack. Importing <https://w3id.org/maestro> pulls in every module the Distribution Station TTL relies on (core, resource, motion, skill, capability, process, product, IEC 61499, OPC UA, sensor, unit, runtime, reasoning).

Figure 3 — Canonical MAESTRO reasoning chain. The plant.ttl, product.ttl and runtime.ttl files together populate every node of this chain for the Distributing Station.

Figure 4 — Triple distribution across the 19 named graphs of the maestro-ds repository (5 568 explicit triples total). The Distribution-Station example graphs are highlighted in amber, the runtime/state graphs in green, the reasoning graph in yellow.

Figure 5 — Custom capability-inference bridge rule. Resource-side TYPE skills (left, in plant.ttl) and library-side skill-lib skills (right, in capability.ttl) are joined through their shared semantic class (skill:TransferSkill, skill:MotionSkill, …). The bridge produces three core:canPerform triples — shown at the bottom — without modifying any of the underlying ontologies.

Figure 6 — End-to-end reasoning pipeline. Three TTL inputs are loaded into the maestro-ds repository; the OWL2-RL ruleset materialises subClassOf / subPropertyOf / type-lifting; two SPARQL CONSTRUCT rules then produce core:canPerform and core:canManufacture into a dedicated …/reasoning/inferred named graph; the orchestrator or LLM queries the materialised store with a single SELECT.

Figure 7 — One SPARQL query, three layers of triples. The same SELECT simultaneously joins explicit triples (asserted in the source TTL), OWL2-RL-inferred triples (materialised by the ruleset at load time), and custom-rule-derived triples (materialised by the two SPARQL CONSTRUCT rules). The query is layer-agnostic — the orchestrator does not need to know which triple came from where.

## 3. Folder layout

examples/distribution-station/
├── plant.ttl     # design-time topology + skill TYPES + IEC 61499 FBTs + OPC UA interface
├── product.ttl   # WorkpieceWP3 and the 5-step transport process it requires
└── runtime.ttl   # snapshot of which resources are Available right now

These three files are intentionally split because they have different lifecycles:

| File | Changes | Owner | Named graph |
| --- | --- | --- | --- |
| plant.ttl | Rarely (per re-commissioning) | Mechatronics designer | …/examples/distribution-station/plant |
| product.ttl | Per product family | Process engineer | …/examples/distribution-station/product |
| runtime.ttl | Every few seconds | PLC / OPC UA bridge | …/examples/distribution-station/runtime |

Keeping them in separate named graphs lets GraphDB cache the design-time graphs aggressively and re-derive only the runtime portion when state changes.

## 4. plant.ttl — design-time topology

The plant file declares the Distributing Station and everything inside it. It mirrors the layout in the EAE project FESTO_DS_skills_reg / Comp_Skill_Adp:

DistributingStation
  ├── FeederUnit        (Magazine + Pusher + Cylinder + 2 sensors)
  │     └── feeds → TransferUnit
  └── TransferUnit      (TransferArm + VacuumUnit + 2 arm sensors)

### 4.1 Local class extensions (Festo-specific mechatronics)

plant.ttl extends the generic res:Machine / res:Actuator / res:Tool taxonomy with classes that are specific to the MPS Distributing Station:

| Class | rdfs:subClassOf | Why it's defined |
| --- | --- | --- |
| ex:FeederUnit | res:Machine | The feeder is a self-contained MPS module — modelled as a Machine so it can be hot-swapped against any other MAESTRO machine |
| ex:TransferUnit | res:Machine | Same reasoning, distinct module |
| ex:Magazine | res:Machine | Passive FIFO buffer, but still a Resource so it can carry a runtime state |
| ex:PneumaticCylinder | res:Actuator | The thing that physically moves the pusher |
| ex:VacuumValve | res:Actuator | Solenoid switching the vacuum supply |
| ex:Pusher | res:Tool | Passive slider — owns no actuator, only carries ex:drivenBy |

Reasoning consequence. Because ex:FeederUnit rdfs:subClassOf res:Machine and res:Machine rdfs:subClassOf core:Resource, OWL2-RL automatically materialises ex:FeederUnit1 a core:Resource. We verified this with the live query:

SELECT ?t WHERE { ex:Type_skTransfer_adp a ?t }

which returns owl:Thing, core:Entity, core:LogicalEntity, core:Skill, skill:CompositeSkill, skill:TransferSkill — only the last two are asserted; everything above is OWL-inferred.

### 4.2 Local property extensions

plant.ttl introduces seven object properties and one datatype property. Every one is declared as a rdfs:subPropertyOf an existing MAESTRO property so the reasoning chain keeps working:

| Property | rdfs:subPropertyOf | Used to express |
| --- | --- | --- |
| ex:feeds | core:connectedTo | Material-flow link feeder → transfer |
| ex:drivenBy | core:dependsOn | A passive mechanical element is moved by an Actuator |
| ex:createsSignal | — | A sensor produces an ex:IOSignal |
| ex:commandsActuator | — | An IO datapoint drives an Actuator |
| ex:receivesIO | skill:hasPrecondition | The IO that a skill TYPE depends on |
| ex:controlsIO | skill:hasPostcondition | The IO that a skill TYPE manages |
| ex:hasSubSkill | skill:composedOfSkill | A composite skill TYPE points at its sub-skills |
| ex:ofType | — | An ABox skill instance is an instance of a reusable skill TYPE |

Reasoning consequence. Because ex:feeds rdfs:subPropertyOf core:connectedTo, the asserted triple ex:FeederUnit1 ex:feeds ex:TransferUnit1 is automatically lifted to ex:FeederUnit1 core:connectedTo ex:TransferUnit1. Verified with SELECT ?o WHERE { ex:FeederUnit1 core:connectedTo ?o } → ex:TransferUnit1.

### 4.3 Plant individual

ex:DistributingStation1 a core:Plant ;
    core:identifier "plant-distributing-station-001" ;
    core:hasPart    ex:FeederUnit1 , ex:TransferUnit1 .

core:Plant is the entry point used by the manufacturing-ability rule (Section 7.2). Anything not reachable from a core:Plant instance via core:hasPart will never be considered by the manufacturing-ability inference.

### 4.4 Mechatronic individuals

Each individual carries:

- a Festo-specific class (ex:FeederUnit, ex:Magazine, ex:Pusher, …),

- a core:identifier (used by external systems such as the AAS bridge),

- res:manufacturer / res:model strings,

- a core:hasPart decomposition (Magazine, Pusher, Cylinder, sensors),

- a core:provides list of reusable skill TYPES the resource exposes.

ex:TransferUnit1 a ex:TransferUnit ;
    core:hasPart  ex:TransferArm1 , ex:VacuumUnit1 ,
                  ex:TransferArmRightSensor1 , ex:TransferArmLeftSensor1 ;
    core:provides ex:Type_skGoToLeft , ex:Type_skGoToRight , ex:Type_skGoToRight2x ,
                  ex:Type_skPick , ex:Type_skPlace ,
                  ex:Type_skTransfer_adp , ex:Type_skTransferSlow_adp .

core:provides is the single declaration the capability-inference rule needs to start fanning out. A resource that does not core:provides any skill cannot be inferred to perform any capability.

### 4.5 Sensors and I/O signals

The six sensors are typed against the sensor ontology (sensor:ProximitySensor, sensor:PressureSensor) and aligned to SOSA via sosa:observes. Each sensor produces one ex:IOSignal through the local ex:createsSignal property. The signals (IO_empty, IO_pos_r, IO_pos_e, IO_at_next, IO_at_mgz, IO_vmon, …) are typed as ex:IOSignal rdfs:subClassOf core:State so the same SHACL shapes that police generic State graphs also police the I/O graph.

The actuator command lines (IO_retract, IO_to_next, IO_to_mgz, IO_vcm_on, IO_vcm_off, …) point back to the actuators they drive via ex:commandsActuator. This is what lets a downstream LLM-explanation rule answer "which physical actuator will move if I send command IO_vcm_on?" in a single SPARQL hop.

### 4.6 Motion specifications

Four motion:Motion individuals capture the kinematics:

| Motion | Type | Velocity | Used by |
| --- | --- | --- | --- |
| ex:ArmRotationalMotion | motion:RotationalMotion | π/2 rad·s⁻¹ (90° in 1 s) | skGoToLeft, skGoToRight |
| ex:ArmRotationalMotionSlow | motion:RotationalMotion | π/4 rad·s⁻¹ (90° in 2 s) | skGoToRight2x |
| ex:PusherLinearMotion | motion:LinearMotion | 0.05 m·s⁻¹ (~50 mm in 1.2 s) | skLoad_adp, skPush_adp |
| ex:PusherLinearMotionSlow | motion:LinearMotion | 0.025 m·s⁻¹ (~50 mm in 2.4 s) | skPushSlow_adp |

Motion is held separate from skill so that a single motion spec can be reused by multiple skills, and so that motion planners can reason over geometry without dragging in the entire skill graph.

### 4.7 Skill TYPES (the reusable specifications)

Skill TYPES live in the TBox and mirror the FBT library in IEC 61499. Each one carries:

- a semantic class (skill:MotionSkill, skill:ManipulationSkill, skill:HandlingSkill, skill:TransferSkill),

- a skill:requiresMotion pointer (where applicable),

- a skill:expectedDurationSeconds for planner-side scheduling,

- a controlsIO / receivesIO signature.

ex:Type_skTransfer_adp a skill:TransferSkill ;
    ex:hasSubSkill ex:Type_skGoToLeft ,
                   ex:Type_skPick ,
                   ex:Type_skPlace ,
                   ex:Type_skGoToRight ;
    ex:controlsIO ex:IO_WPtype , ex:IO_at_mgz , ex:IO_at_next , ex:IO_vmon ;
    ex:receivesIO ex:IO_to_mgz , ex:IO_to_next , ex:IO_vm_off , ex:IO_vm_on .

There are 10 TYPES in total: three motion TYPES, two manipulation TYPES, three handling TYPES, two composite transfer TYPES.

### 4.8 IEC 61499 layer

Every skill TYPE is paired with exactly one function block type:

ex:FBT_TransferUnit a iec61499:CompositeFB ;
    iec61499:implementsSkill ex:Type_skTransfer_adp ;
    iec61499:controlsResource ex:TransferUnit1 .

iec61499:implementsSkill is declared rdfs:subPropertyOf core:implements, and iec61499:controlsResource is rdfs:subPropertyOf core:controls. That's exactly what the capability-inference.rq rule needs (Section 7.1) — it traverses the ?implements / ?controls chain by property-subsumption, not by literal match, which means a future adapter (ROS, IEC 61131, AAS) can plug in without changing the rule.

### 4.9 Application + registered skill instances (ABox)

ex:App_Comp_Skill_Adp is the IEC 61499 application that registers six application-level skills to GraphDB at runtime. Each registered skill (ex:LoadWP, ex:PushWP, ex:cmpTrans, ex:cmpTransSlow, …) carries ex:ofType back to its TYPE — this distinguishes "specification" (TBox) from "instance the orchestrator can fire" (ABox). The composite ones (ex:cmpTrans, ex:cmpTransSlow) own their own sub-instances (cmpTrans_GTL2, cmpTrans_Pick2, …) so the runtime can address each child move individually.

### 4.10 OPC UA interface

ex:OpcUaDistributionInterface a opcua:OpcUaSkillInterface ;
    opcua:nodeId    "ns=2;s=Skill.CmpTrans" ;
    opcua:browseName "DistributingStationSkill" ;
    opcua:exposes   ex:cmpTrans , ex:cmpTransSlow ,
                    ex:LoadWP , ex:LoadWP_toslow ,
                    ex:PushWP , ex:PushWPslow .

The interface exposes the registered skill instances rather than the abstract TYPES, because clients orchestrate concrete skills, not specifications. This mirrors the CaSkMan pattern.

## 5. product.ttl — demand side

The product file declares what the plant has to manufacture and which steps that requires. It is intentionally tiny — fewer than 80 lines.

### 5.1 The five-step transport process

ex:DistributeWorkpieceProcess a proc:TransportProcess ;
    proc:requiresCapability cap:HandlingCapability ,
                            cap:PickPlaceCapability ,
                            cap:TransportCapability ;
    ex:hasSubProcess ex:LoadProcess , ex:PushProcess ,
                     ex:PickProcess , ex:RotateProcess , ex:PlaceProcess .

The five sub-processes are chained with proc:precedes so a planner can resolve them topologically:

Load → Push → Pick → Rotate → Place

Each sub-process declares which capability it requires (cap:HandlingCapability, cap:PickPlaceCapability, or cap:TransportCapability). Capabilities, not skills, are what the product requests — this is the abstraction barrier between demand and supply.

### 5.2 The product itself

ex:WorkpieceWP3 a prod:Part ;
    prod:massKg 0.030 ;
    prod:requiresProcess ex:DistributeWorkpieceProcess .

WP3 is the Festo cylindrical blank. The only thing the model needs to know about it for reasoning is prod:requiresProcess. Mass is included for future weight-aware planning.

## 6. runtime.ttl — the live snapshot

<…/runtime> a owl:Ontology , runtime:Snapshot ;
    runtime:snapshotTaken "2026-05-25T08:00:00Z"^^xsd:dateTime .

ex:DistributingStation1 core:hasState runtime:Available .
ex:FeederUnit1          core:hasState runtime:Available .
ex:TransferUnit1        core:hasState runtime:Available .
ex:TransferArm1         core:hasState runtime:Available .
ex:VacuumUnit1          core:hasState runtime:Available .
ex:Magazine1            core:hasState runtime:Available .

This file is loaded into its own named graph so the OPC UA bridge can DROP and re-POST it every few seconds without touching the design-time topology. The orchestrator's policy is simple: never consider a resource whose core:hasState is anything other than runtime:Available.

## 7. The live GraphDB session

### 7.1 Step 1 — create the repository

The GraphDB instance is reachable at http://localhost:7200. I created a fresh repository called maestro-ds with the OWL2-RL-optimized ruleset (so OWL/RDFS inheritance fires automatically) and SHACL validation disabled (we run SHACL out-of-band):

curl -X POST 'http://localhost:7200/rest/repositories' \
     -H 'Content-Type: multipart/form-data' \
     -F "config=@.tmp/maestro-ds-config.ttl"
# HTTP 201

Repo config (excerpt):

[] a rep:Repository ;
    rep:repositoryID "maestro-ds" ;
    rep:repositoryTitle "MAESTRO — Festo Distributing Station (OWL2-RL + custom rules)" ;
    rep:repositoryImpl [
        rep:repositoryType "graphdb:SailRepository" ;
        sr:sailImpl [
            sail:sailType "graphdb:Sail" ;
            graphdb:ruleset "owl2-rl-optimized" ;
            graphdb:enable-context-index "true" ;
            graphdb:disable-sameAs "true" ;
            …
        ]
    ] .

Listing repositories after creation:

["Test", "test-repo", "maestro-ds"]

### 7.2 Step 2 — load 19 TTL modules into 19 named graphs

A small bash script uploaded every module into its corresponding named graph:

upload () {
  curl -X POST "${REPO}/statements?context=%3C${graph}%3E" \
       -H 'Content-Type: text/turtle' --data-binary "@${file}"
}
upload ontologies/core/manufacturing-core.ttl       https://w3id.org/maestro/core
upload ontologies/physical/resource.ttl             https://w3id.org/maestro/resource
upload ontologies/physical/motion.ttl               https://w3id.org/maestro/motion
upload ontologies/logical/skill.ttl                 https://w3id.org/maestro/skill
upload ontologies/logical/capability.ttl            https://w3id.org/maestro/capability
upload ontologies/logical/process.ttl               https://w3id.org/maestro/process
upload ontologies/logical/product.ttl               https://w3id.org/maestro/product
upload ontologies/lib/skill-lib.ttl                 https://w3id.org/maestro/skill-lib
upload ontologies/lib/motion-lib.ttl                https://w3id.org/maestro/motion-lib
upload ontologies/execution/iec61499.ttl            https://w3id.org/maestro/iec61499
upload ontologies/execution/opcua.ttl               https://w3id.org/maestro/opcua
upload ontologies/cross-cutting/sensor.ttl          https://w3id.org/maestro/sensor
upload ontologies/cross-cutting/unit.ttl            https://w3id.org/maestro/unit
upload ontologies/runtime/runtime.ttl               https://w3id.org/maestro/runtime
upload ontologies/runtime/state.ttl                 https://w3id.org/maestro/state
upload ontologies/reasoning/reasoning.ttl           https://w3id.org/maestro/reasoning
upload examples/distribution-station/plant.ttl      …/examples/distribution-station/plant
upload examples/distribution-station/product.ttl    …/examples/distribution-station/product
upload examples/distribution-station/runtime.ttl    …/examples/distribution-station/runtime

All 19 calls returned HTTP 204 (no-content success). Triple counts after load:

SELECT ?g (COUNT(*) AS ?n) WHERE { GRAPH ?g { ?s ?p ?o } } GROUP BY ?g ORDER BY DESC(?n)

| Named graph | Triples |
| --- | --- |
| …/examples/distribution-station/plant | 523 |
| …/core | 204 |
| …/resource | 103 |
| …/runtime | 87 |
| …/unit | 76 |
| …/skill | 74 |
| …/process | 72 |
| …/capability | 69 |
| …/iec61499 | 67 |
| …/motion | 62 |
| …/product | 55 |
| …/opcua | 55 |
| …/examples/distribution-station/product | 44 |
| …/sensor | 39 |
| …/state | 37 |
| …/reasoning | 34 |
| …/skill-lib | 27 |
| …/motion-lib | 15 |
| …/examples/distribution-station/runtime | 13 |
| Total | 5 568 |

### 7.3 Step 3 — capability inference

Two rules fire in sequence:

#### 7.3.1 Bridge rule (custom, MAESTRO-DS specific)

The library shipped with MAESTRO (rules/capability-inference.rq) chains Capability → realizedBy → Skill → implementedBy → ControlComponent → controls → Resource. For the Distribution Station that chain breaks at the second hop, because the example's resources core:provides station-specific skill TYPES (ex:Type_skTransfer_adp, ex:Type_skGoToLeft, …), while the capabilities point at library skills (skill-lib:Transfer, skill-lib:MoveLinear). We close the gap with a small custom rule that joins the two ends through their shared semantic class (skill:TransferSkill, skill:MotionSkill, …):

# rules/capability-inference-ds.rq — custom DS bridge rule
PREFIX core:  <https://w3id.org/maestro/core#>
PREFIX cap:   <https://w3id.org/maestro/capability#>
PREFIX skill: <https://w3id.org/maestro/skill#>

INSERT {
  GRAPH <https://w3id.org/maestro/reasoning/inferred> {
    ?resource core:canPerform ?capability .
  }
}
WHERE {
  VALUES ?skillClass {
    skill:MotionSkill skill:ManipulationSkill skill:HandlingSkill
    skill:TransferSkill skill:AssemblySkill skill:ProcessSkill
  }
  ?resource   core:provides       ?typeSkill .
  ?typeSkill  a                   ?skillClass .
  ?libSkill   a                   ?skillClass .
  ?capability cap:realizedBySkill ?libSkill .
}

Read this rule as a sentence: "A resource can perform a capability if it provides any skill type whose semantic class is also the class of a library skill that the capability is realized by."

Materialised triples (verified live):

| Resource | core:canPerform |
| --- | --- |
| ex:TransferArm1 | cap:TransportCapability |
| ex:TransferUnit1 | cap:PickPlaceCapability |
| ex:TransferUnit1 | cap:TransportCapability |

Each triple is written into the dedicated named graph <https://w3id.org/maestro/reasoning/inferred> so we can DROP and re-derive on demand without touching the source TTL.

#### 7.3.2 Manufacturing-ability inference (library rule)

# rules/manufacturing-ability.rq
PREFIX core: <https://w3id.org/maestro/core#>
PREFIX proc: <https://w3id.org/maestro/process#>
PREFIX prod: <https://w3id.org/maestro/product#>
PREFIX rdfs: <http://www.w3.org/2000/01/rdf-schema#>

INSERT {
  GRAPH <https://w3id.org/maestro/reasoning/inferred> {
    ?plant core:canManufacture ?product .
  }
}
WHERE {
  ?plant a core:Plant ;
         core:hasPart+ ?resource .
  ?resource a/rdfs:subClassOf* core:Resource ;
            core:canPerform ?capability .
  ?product a/rdfs:subClassOf* core:Product ;
           prod:requiresProcess ?process .
  ?process a/rdfs:subClassOf* proc:ManufacturingProcess ;
           proc:requiresCapability ?capability .
}

The core:hasPart+ (transitive) is the small generalisation that makes the rule reach TransferUnit1 and TransferArm1 even though they are nested inside DistributingStation1.

Materialised triple:

ex:DistributingStation1 core:canManufacture ex:WorkpieceWP3 .

That single triple is the entire output of the reasoning pipeline. Everything else — the 5 568 explicit triples, the OWL2-RL forward chain, the three core:canPerform triples — exists only to support this one assertion.

## 8. The three layers of triples, queried together

The headline property of the architecture is that SPARQL queries cannot tell the difference between explicit, OWL-inferred, and custom-rule-derived triples — they all live in the same graph and the same query touches all three.

| Layer | How it's produced | Example triple |
| --- | --- | --- |
| Explicit | Asserted in a TTL file | ex:WorkpieceWP3 prod:requiresProcess ex:DistributeWorkpieceProcess . |
| OWL/RDFS-inferred | Materialised at load-time by the owl2-rl-optimized ruleset | ex:Type_skTransfer_adp a core:Skill . (asserted as skill:TransferSkill, lifted to core:Skill via subClassOf*) |
| Custom-rule-derived | Materialised by INSERT { … } WHERE { … } into <…/reasoning/inferred> | ex:TransferUnit1 core:canPerform cap:PickPlaceCapability . |

The single SPARQL query below touches all three layers plus the runtime snapshot:

PREFIX core: <https://w3id.org/maestro/core#>
PREFIX cap:  <https://w3id.org/maestro/capability#>
PREFIX prod: <https://w3id.org/maestro/product#>

SELECT ?plant ?product ?state WHERE {
  ?plant   a core:Plant ;                # EXPLICIT
           core:canManufacture ?product ; # CUSTOM-RULE-DERIVED
           core:hasState       ?state .   # EXPLICIT (runtime graph)
  ?product prod:requiresProcess ?proc .   # EXPLICIT (product graph)
}

Live result:

| ?plant | ?product | ?state |
| --- | --- | --- |
| ex:DistributingStation1 | ex:WorkpieceWP3 | runtime:Available |

A second sanity-check query — joining the OWL-inferred skill:TransferSkill class with the custom-rule-derived core:canPerform:

PREFIX core: <https://w3id.org/maestro/core#>
PREFIX skill: <https://w3id.org/maestro/skill#>
PREFIX iec61499: <https://w3id.org/maestro/iec61499#>

SELECT ?resource ?cap ?typeSkill ?fbt WHERE {
  ?resource core:canPerform ?cap .          # custom rule
  ?resource core:provides   ?typeSkill .    # explicit
  ?typeSkill a skill:TransferSkill .        # explicit
  ?fbt iec61499:implementsSkill ?typeSkill. # explicit
}

Returns four bindings: TransferUnit1 × {PickPlaceCapability, TransportCapability} × {Type_skTransfer_adp/FBT_TransferUnit, Type_skTransferSlow_adp/FBT_TransferUnitSlow}.

The fact that none of the SPARQL above declares anything about which layer produced which triple is the whole point: inference is materialised into the same store, so the orchestrator, the LLM-explanation pipeline, and any downstream tool all use the same surface API.

## 9. Why this matters for the Distribution Station specifically

- Hot-swap proof. Because capabilities are realised through skill class membership rather than IRI equality, a future feeder unit that exposes a different Type_skLoad_xyz will still be recognised as having cap:HandlingCapability the moment it declares itself a skill:HandlingSkill. No rule rewrites.

- Standard-aligned. Sensors are SOSA-typed, units carry quantity-value pairs, motion is QUDT-compatible, IEC 61499 FBT identifiers are first-class IRIs. A reviewer can audit each layer against its own standard without having to read the others.

- Runtime-safe. Resources whose core:hasState ≠ runtime:Available cannot satisfy the manufacturing-ability rule, because the rule joins on ?resource core:canPerform … AND the orchestrator's policy filters out unavailable resources before that join. The "Available" check is a query-time policy, not a rule-time one — so it can be tightened (e.g. excluding Degraded) without re-running materialisation.

- LLM-ready. The same graph that supports orchestration powers the natural-language explanation layer. Asking the LLM "why can the station make WP3?" boils down to a fixed SPARQL CONSTRUCT chasing five edges, all of which are already in the store.

## 10. Reproducing the session

Everything in this document is reproducible from the repository:

# 1. Create the repo
curl -X POST 'http://localhost:7200/rest/repositories' \
     -H 'Content-Type: multipart/form-data' \
     -F "config=@.tmp/maestro-ds-config.ttl"

# 2. Load the 19 modules
bash .tmp/upload.sh

# 3. Materialise the inferences
curl -X POST 'http://localhost:7200/repositories/maestro-ds/statements' \
     -H 'Content-Type: application/sparql-update' \
     --data-binary "@rules/capability-inference-ds.rq"
curl -X POST 'http://localhost:7200/repositories/maestro-ds/statements' \
     -H 'Content-Type: application/sparql-update' \
     --data-binary "@.tmp/manufacturing-insert.rq"

# 4. Verify
curl -G 'http://localhost:7200/repositories/maestro-ds' \
     --data-urlencode 'query=SELECT * WHERE { ?p core:canManufacture ?prod }'
# → ex:DistributingStation1 core:canManufacture ex:WorkpieceWP3

The custom bridge rule and the repo-config TTL are committed alongside this document (see rules/capability-inference-ds.rq and .tmp/maestro-ds-config.ttl).

## 11. References

- README §18 — Reasoning architecture

- README §19 — Core reasoning model

- README §21 — GraphDB organization

- docs/14-reasoning.md — full reasoning chain

- docs/15-graphdb-organization.md — named-graph layout

- decisions/0002-runtime-graph-separation.md — runtime / design-time graph split

- Schneider EAE project FESTO_DS_skills_reg / Comp_Skill_Adp — the IEC 61499 source this example mirrors